Ophio

# Features

Ophio is a 4-channel Gigabit optical receiver which couples to the Chimaera module.

The optical receiver module is a 12-channel Agilent or Emcore part of which channels 8 to 11 are decoded using Texas deserialisers. The module requires a 5V power supply which is usually provided through the Chimaera coupling connector but can also be provided by a bench top supply for standalone tests.

# Connectors and indicators

## Status LEDs

A bank of eight LEDs are provided at the front panel. These are connected to pins of the Chimaera FPGA through the coupling connector to provide programmable functionality.

## Two pole LEMO00 front panel connector

This connector is coupled to a pair of Chimaera FPGA IO pins via the coupling connector. The pins may be programmed by the application as either single ended or differential IO. The Chimaera board has provision for mounting terminating resistors when the connector is configured as an input.

## Front-end coupling connector, SKT1

Ophio is normally connected to the Chimaera. This connector provides power to Ophio and links Chimaera FPGA IO pins to the Ophio signals.