The Chimaera

# Features

The back-end half of the board comprises one Spartan2e FPGA (xc2s200e), a flash memory (xc18v02) for non-volatile storage of the FPGA bitstream and one 256Mbit SDRAM. There is an on-board 80MHz crystal for those applications that require it.

The board is mechanically and electrically compatible with the S-link standard.

Power supplies are picked up either from mother board: +5V (raw, through S-link connector), Vaux (through auxiliary power pins SKT2). 2mm sockets for the 5V supply are also provided for standalone or testbench use. Vaux is not used by the back-end but is routed to the front-end coupling connector. There is no on-board over-voltage or reverse voltage protection.

The board implements a JTAG TAP for FPGA configuration.

# FPGA configuration

Both the FPGA and its configuration flash memory are connected to the JTAG TAP. A bitstream may be directly loaded into the FPGA or downloaded into the flash memory from where it is downloaded to the FPGA on pressing SW1 or power cycling the board.

# Connectors, switches and indicators

## Program switch, SW1

SW1 triggers the reset of the FPGA. If “master serial” mode is selected and there is a valid bitstream in the configuration flash memory it is reloaded into the FPGA.

## Configuration mode switch, SW2

SW2 is used to select between FPGA “boundary scan” or “master serial” configuration mode. The switch can normally be left in the “master serial” position as “boundary scan” (i.e. JTAG) configuration is also possible in this mode.

## 5V power and ground, TP1 and TP2

When not plugged into a motherboard it may be convenient to provide 5V power and ground to the board using these 2mm sockets.

## Auxiliary power connector, SKT2

The S-link standard connector provides for only one power supply. For additional flexibility, socket SK2 is provided which mates with corresponding pins on the motherboard or can be connected to an auxiliary supply to provide an additional power voltage. The voltage can be any value and any polarity (it is not used on the back-end board) but must match the polarity requirements of the front-end half of the board to which it is connected through the front-end coupling connector.

## S-link connector, PL1

This board is configured as an S-link LDC. The board can be used as a non-standard LSC provided care is taken with pin 5 of PL1. In standard usage pin 5 of an LDC is used for LDERR_B while for an LSC it is Vcc. Provided the FPGA IO J19 is not configured as a driver the board should also work as an LSC having J19 connected directly to Vcc. If J19 is driving in this situation there is the possibility of signal contention which might result in permanent damage to the FPGA.

## Front-end coupling connector, SKT1

The back-end board is normally connected to a front-end board. This connector provides power to the front-end board and links FPGA IO to signals on the front-end board.

## Program indicator LED1

This LED normally lights briefly when PROGRAM is asserted (e.g. by pressing switch SW1).

## Done indicator LED2

The LED normally lights when the FPGA is not configured (actually indicates NOT DONE). After successful configuration of the FPGA the LED should be off. The LED will remain on after pressing SW1 if the FPGA is in “boundary scan” configuration mode and the bitstream has not yet been downloaded or in “master serial” mode if there is no valid bitstream in the configuration flash memory.